# Circuit Wiring:

## ⚡ POWER DISTRIBUTION SYSTEM

### 12V Battery & Buck Converter

12V Li-ion Battery:

├─ Positive (+) →┬→ Buck Converter VIN+

│                ├→ L298N Motor Driver +12V

│                └→ Relay Module COM (Common)

│

└─ Negative (-) → COMMON GROUND BUS

Buck Converter (12V to 5V):

├─ VIN+ → 12V Battery (+)

├─ VIN- → Common Ground

├─ VOUT+ (5V) → Raspberry Pi Pin 2 (5V)

│              → Raspberry Pi Pin 4 (5V)

│              → Breadboard 5V Rail

└─ VOUT- → Common Ground

### Common Ground Bus (Connect ALL Together)

★ CRITICAL: All grounds must be connected ★

Common Ground includes:

├─ 12V Battery (-)

├─ Buck Converter VIN-

├─ Buck Converter VOUT-

├─ Raspberry Pi Pin 6, 9, 14, 20, 25, 30, 34, 39 (GND)

├─ L298N GND

├─ Relay Module GND

├─ 12V Fan (-)

├─ Breadboard Ground Rail (-)

└─ All sensor grounds (see below)

## 🖥️ RASPBERRY PI 4B PIN CONNECTIONS

### Power Pins

Pin 1  (3.3V) → ADS1115 VDD ★ CRITICAL: Use 3.3V not 5V ★

Pin 2  (5V)   → Buck Converter VOUT+ (5V)

Pin 4  (5V)   → Breadboard 5V Rail (+)

Pin 6  (GND)  → Buck Converter VOUT- & Common Ground

Pin 9  (GND)  → Breadboard Ground Rail (-)

Pin 14 (GND)  → Relay Module GND

### I2C Communication

Pin 3 (GPIO 2 - SDA) → ADS1115 SDA

Pin 5 (GPIO 3 - SCL) → ADS1115 SCL

### Environmental Sensors

Pin 7  (GPIO 4)  → DHT22 DATA

Pin 11 (GPIO 17) → 150Ω Resistor → GP2Y1010F48 Pin 1 (V-LED)

Pin 15 (GPIO 22) → Relay Module IN1

### Motor Driver (L298N)

Pin 33 (GPIO 13) → L298N ENA (Left Motor Enable)

Pin 35 (GPIO 19) → L298N IN1 (Left Motor Direction)

Pin 37 (GPIO 26) → L298N IN2 (Left Motor Direction)

Pin 12 (GPIO 18) → L298N ENB (Right Motor Enable)

Pin 38 (GPIO 20) → L298N IN3 (Right Motor Direction)

Pin 40 (GPIO 21) → L298N IN4 (Right Motor Direction)

### Ultrasonic Sensors

Pin 16 (GPIO 23) → Ultrasonic Front TRIG

Pin 18 (GPIO 24) → Ultrasonic Front ECHO

Pin 22 (GPIO 25) → Ultrasonic Side TRIG

Pin 24 (GPIO 8)  → Ultrasonic Side ECHO

### IR Obstacle Sensors

Pin 26 (GPIO 7)  → IR Left OUT

Pin 32 (GPIO 12) → IR Right OUT

## 📟 ADS1115 ADC MODULE (16-bit Analog to Digital Converter)

★ CRITICAL CORRECTION: Use 3.3V power for I2C safety ★

ADS1115 Connections:

├─ VDD  → Raspberry Pi Pin 1 (3.3V) ← MUST BE 3.3V

├─ GND  → Common Ground / Breadboard Ground Rail

├─ SCL  → Raspberry Pi Pin 5 (GPIO 3)

├─ SDA  → Raspberry Pi Pin 3 (GPIO 2)

├─ ADDR → GND (sets I2C address to 0x48)

│

└─ Analog Inputs:

├─ A0 → MQ-7 AOUT (CO sensor)

├─ A1 → MQ-135 AOUT (Air quality sensor)

├─ A2 → GP2Y1010F48 Pin 5 (Vo output) ← See PM2.5 section

└─ A3 → Not connected

## 🌡️ DHT22 TEMPERATURE/HUMIDITY SENSOR

DHT22 4-Pin Sensor:

├─ Pin 1 (VCC)  → Breadboard 5V Rail

├─ Pin 2 (DATA) → Raspberry Pi Pin 7 (GPIO 4)

├─ Pin 3 (NC)   → Not connected

└─ Pin 4 (GND)  → Breadboard Ground Rail

Note: Some DHT22 modules have built-in pull-up resistor

If using bare sensor, add 10kΩ resistor between DATA and VCC

## 💨 MQ-7 CO (Carbon Monoxide) SENSOR

MQ-7 Module (4 pins):

├─ VCC  → Breadboard 5V Rail

├─ GND  → Breadboard Ground Rail

├─ DOUT → Not connected (not used in code)

└─ AOUT → ADS1115 Channel A0

⚠️ IMPORTANT:

- Requires 24-48 hour warm-up for accurate readings

- Code uses 30-second minimum warm-up

- Keep away from direct airflow

## 💨 MQ-135 AIR QUALITY SENSOR (VOC/NH3/CO2)

MQ-135 Module (4 pins):

├─ VCC  → Breadboard 5V Rail

├─ GND  → Breadboard Ground Rail

├─ DOUT → Not connected (not used in code)

└─ AOUT → ADS1115 Channel A1

⚠️ IMPORTANT:

- Requires 24-48 hour warm-up for accurate readings

- Code uses 30-second minimum warm-up

- Sensitive to temperature and humidity (code compensates)

## 💨 GP2Y1010F48 PM2.5 DUST SENSOR

★ CRITICAL CORRECTIONS APPLIED ★

GP2Y1010F48 Standard Pinout (6 pins):

Pin 1 (V-LED)   → 150Ω Resistor → GPIO 17 (Pi Pin 11)

Pin 2 (LED-GND) → Common Ground

Pin 3 (LED)     → Breadboard 5V Rail

Pin 4 (S-GND)   → Common Ground

Pin 5 (Vo)      → 220µF Capacitor (+) → ADS1115 A2

Pin 6 (Vcc)     → Breadboard 5V Rail

Components Required:

├─ 150Ω Resistor (1/4W): GPIO 17 → Pin 1

└─ 220µF Capacitor: Pin 5 (Vo) to GND

220µF Electrolytic Capacitor Connection:

├─ Positive (+) Long Leg  → Pin 5 (Vo) AND ADS1115 A2

└─ Negative (-) Short Leg → Common Ground

⚠️ POLARITY MATTERS:

- Capacitor wrong polarity = sensor damage

- LED control via Pin 1, not Pin 3

- Pin 5 is OUTPUT, Pin 6 is POWER

### Breadboard Wiring for PM2.5 (Detailed)

5V Rail ────┬─────────────────→ Pin 3 (LED anode)

└─────────────────→ Pin 6 (Vcc power)

GPIO 17 ────[150Ω Resistor]──→ Pin 1 (V-LED cathode control)

GND Rail ───┬─────────────────→ Pin 2 (LED-GND)

├─────────────────→ Pin 4 (S-GND)

└───[220µF (-)]

Pin 5 (Vo) ─[220µF (+)]───────→ ADS1115 A2

This allows GPIO control: LOW = LED ON for measurement

## 🚗 L298N MOTOR DRIVER MODULE

### Power Connections

L298N Power:

├─ +12V    → 12V Battery (+)

├─ GND     → Common Ground

├─ +5V Out → NOT USED (Remove jumper if present)

└─ Jumper  → REMOVE (to prevent backfeeding 5V to Pi)

⚠️ Remove the 5V enable jumper between +12V and +5V terminals

### Left Motor Control (Motor A)

L298N Motor A:

├─ ENA (Enable A) → GPIO 13 (Pi Pin 33) - PWM for speed

├─ IN1            → GPIO 19 (Pi Pin 35) - Direction bit 1

├─ IN2            → GPIO 26 (Pi Pin 37) - Direction bit 2

├─ OUT1           → Left DC Motor Wire 1

└─ OUT2           → Left DC Motor Wire 2

Motor Direction Logic:

- IN1=LOW, IN2=HIGH → Forward

- IN1=HIGH, IN2=LOW → Backward

- IN1=LOW, IN2=LOW  → Stop/Brake

### Right Motor Control (Motor B)

L298N Motor B:

├─ ENB (Enable B) → GPIO 18 (Pi Pin 12) - PWM for speed

├─ IN3            → GPIO 20 (Pi Pin 38) - Direction bit 1

├─ IN4            → GPIO 21 (Pi Pin 40) - Direction bit 2

├─ OUT3           → Right DC Motor Wire 1

└─ OUT4           → Right DC Motor Wire 2

Motor Direction Logic:

- IN3=LOW, IN4=HIGH → Forward

- IN3=HIGH, IN4=LOW → Backward

- IN3=LOW, IN4=LOW  → Stop/Brake

## 📡 ULTRASONIC SENSORS (HC-SR04)

### Front Ultrasonic Sensor

Ultrasonic Front:

├─ VCC  → Breadboard 5V Rail

├─ TRIG → GPIO 23 (Pi Pin 16)

├─ ECHO → GPIO 24 (Pi Pin 18)

└─ GND  → Breadboard Ground Rail

Function: Detects obstacles ahead

Trigger Distance: 20 cm (OBSTACLE_DISTANCE in code)

### Side Ultrasonic Sensor

Ultrasonic Side:

├─ VCC  → Breadboard 5V Rail

├─ TRIG → GPIO 25 (Pi Pin 22)

├─ ECHO → GPIO 8 (Pi Pin 24)

└─ GND  → Breadboard Ground Rail

Function: Detects obstacles on side

Trigger Distance: 15 cm (SIDE_DISTANCE in code)

## 👁️ IR OBSTACLE SENSORS

### Left IR Sensor

IR Left:

├─ VCC → Breadboard 5V Rail

├─ OUT → GPIO 7 (Pi Pin 26)

└─ GND → Breadboard Ground Rail

Output Logic: LOW = Obstacle detected, HIGH = Clear

Function: Edge/drop detection on left side

### Right IR Sensor

IR Right:

├─ VCC → Breadboard 5V Rail

├─ OUT → GPIO 12 (Pi Pin 32)

└─ GND → Breadboard Ground Rail

Output Logic: LOW = Obstacle detected, HIGH = Clear

Function: Edge/drop detection on right side

## 🔄 4-CHANNEL RELAY MODULE

### Control Side (Low-level trigger)

Relay Module Control:

├─ VCC → Raspberry Pi Pin 4 (5V)

├─ GND → Raspberry Pi Pin 14 (GND) & Common Ground

├─ IN1 → GPIO 22 (Pi Pin 15) - Fan control

├─ IN2 → Not connected (available for future use)

├─ IN3 → Not connected (available for future use)

└─ IN4 → Not connected (available for future use)

Control Logic:

- GPIO HIGH = Relay OFF (NC connected)

- GPIO LOW  = Relay ON (NO connected)

### Switching Side (Channel 1 for Fan)

Relay Channel 1:

├─ COM (Common)     → 12V Battery (+)

├─ NO (Normally Open) → 12V Fan (+) Red Wire

└─ NC (Normally Closed) → Not connected

When GPIO 22 = LOW: Fan receives 12V power (ON)

When GPIO 22 = HIGH: Fan disconnected (OFF)

## 🌀 12V DC FAN (Air Purification)

12V Fan Connections:

├─ (+) Red Wire   → Relay CH1 NO (Normally Open)

└─ (-) Black Wire → 12V Battery (-) / Common Ground

Control Logic in Code:

- AQI > 100: GPIO.output(RELAY_FAN_PIN, GPIO.LOW)  → Fan ON

- AQI ≤ 100: GPIO.output(RELAY_FAN_PIN, GPIO.HIGH) → Fan OFF

## 🔌 BREADBOARD POWER RAILS DISTRIBUTION

### 5V Positive Rail (Red +)

Source: Buck Converter VOUT+ (connected to Pi Pin 4)

5V Rail feeds power to:

├─ MQ-7 VCC

├─ MQ-135 VCC

├─ GP2Y1010F48 Pin 3 (LED) and Pin 6 (Vcc)

├─ DHT22 Pin 1 (VCC)

├─ Ultrasonic Front VCC

├─ Ultrasonic Side VCC

├─ IR Left VCC

├─ IR Right VCC

└─ Relay Module VCC

Total current draw: ~2.5A (ensure buck converter rated ≥3A)

### Ground Rail (Black -)

Source: Raspberry Pi Pin 9 (GND) connected to Common Ground

Ground Rail connects to:

├─ ADS1115 GND

├─ MQ-7 GND

├─ MQ-135 GND

├─ GP2Y1010F48 Pin 2 & Pin 4

├─ 220µF Capacitor (-)

├─ DHT22 Pin 4 (GND)

├─ Ultrasonic Front GND

├─ Ultrasonic Side GND

├─ IR Left GND

├─ IR Right GND

└─ All other ground connections

## 🔧 ASSEMBLY SEQUENCE (Step-by-Step)

### Step 1: Power System

- Connect 12V battery to buck converter VIN+/VIN-

- Set buck converter output to exactly 5.0V (use multimeter)

- Connect buck VOUT+ to Pi Pin 2 and breadboard 5V rail

- Connect all grounds to common ground bus

- Verify voltages BEFORE connecting sensors

### Step 2: ADS1115 & I2C

- Connect ADS1115 VDD to Pi Pin 1 (3.3V) ★ CRITICAL ★

- Connect ADS1115 GND to common ground

- Connect SDA to Pi Pin 3, SCL to Pi Pin 5

- Connect ADDR to GND

- Test: Run i2cdetect -y 1 (should show 0x48)

### Step 3: Gas Sensors

- Connect MQ-7 and MQ-135 to breadboard 5V and GND

- Connect MQ-7 AOUT to ADS1115 A0

- Connect MQ-135 AOUT to ADS1115 A1

- Let sensors warm up (24-48 hours ideal, 30 min minimum)

### Step 4: PM2.5 Sensor (Critical)

- Connect GP2Y1010F48 Pin 6 (Vcc) to 5V rail

- Connect Pins 2 and 4 to GND rail

- Connect Pin 3 (LED) to 5V rail

- Solder 150Ω resistor: GPIO 17 → Pin 1 (V-LED)

- Solder 220µF capacitor: Pin 5 (+) to GND (-)

- Connect Pin 5 AND capacitor (+) to ADS1115 A2

- Double-check capacitor polarity!

### Step 5: DHT22

- Connect to breadboard: VCC→5V, GND→GND, DATA→GPIO 4

### Step 6: Motor System

- Connect L298N +12V to battery (+)

- Remove 5V jumper on L298N

- Connect 6 control pins (ENA, IN1-4, ENB) to Pi

- Connect motors to OUT1-4

- Test motors individually

### Step 7: Navigation Sensors

- Connect both ultrasonics (VCC, TRIG, ECHO, GND)

- Connect both IR sensors (VCC, OUT, GND)

- Test each sensor individually

### Step 8: Relay & Fan

- Connect relay control (VCC, GND, IN1)

- Connect relay COM to battery (+)

- Connect relay NO to fan (+)

- Connect fan (-) to battery (-)

- Test: gpio -g write 22 0 (fan should turn on)

## ✅ PRE-OPERATION TESTING CHECKLIST

### Power System Tests

- [ ] Buck converter outputs exactly 5.0V (±0.1V)

- [ ] No shorts between 5V and GND (use multimeter continuity)

- [ ] Battery voltage ≥ 11.5V (check with multimeter)

- [ ] All ground connections common (0Ω resistance between any two grounds)

### I2C & Sensors Tests

# Install I2C tools

sudo apt-get install i2c-tools

# Check I2C devices

i2cdetect -y 1

# Should show device at address 0x48 (ADS1115)

# Test ADS1115 channels

python3 -c "

import board, busio

import adafruit_ads1x15.ads1115 as ADS

from adafruit_ads1x15.analog_in import AnalogIn

i2c = busio.I2C(board.SCL, board.SDA)

ads = ADS.ADS1115(i2c)

print(f'A0: {AnalogIn(ads, 0).voltage}V')

print(f'A1: {AnalogIn(ads, 1).voltage}V')

print(f'A2: {AnalogIn(ads, 2).voltage}V')

"

### GPIO Tests

# Test PM2.5 LED control

gpio -g mode 17 out

gpio -g write 17 0  # Should enable LED measurement

gpio -g write 17 1  # Should disable

# Test relay/fan

gpio -g mode 22 out

gpio -g write 22 0  # Fan should turn ON

gpio -g write 22 1  # Fan should turn OFF

# Test motor enable pins

gpio -g mode 13 pwm

gpio -g pwm 13 512  # 50% duty cycle

### Sensor Verification

- [ ] DHT22 returns temperature/humidity (test with simple script)

- [ ] MQ-7 voltage changes when exposed to breath

- [ ] MQ-135 voltage changes with air quality

- [ ] PM2.5 shows voltage spike when LED pulses

- [ ] Ultrasonics return distances (test with hand)

- [ ] IR sensors trigger when surface detected

### Motor Tests

- [ ] Left motor forward/backward/stop works

- [ ] Right motor forward/backward/stop works

- [ ] PWM speed control functional (0-100%)

- [ ] Motors don't draw excessive current (check with multimeter)

## ⚠️ CRITICAL SAFETY WARNINGS

### Electrical Safety

❌ NEVER connect 12V directly to Raspberry Pi GPIO pins

❌ NEVER connect 5V to ADS1115 VDD (use 3.3V only)

❌ NEVER reverse battery polarity

❌ NEVER run motors without heat sinking L298N

✅ ALWAYS double-check polarity before powering on

✅ ALWAYS use proper gauge wire (18-22 AWG for 12V)

✅ ALWAYS monitor battery voltage (stop at 10.5V)

### Component Protection

- Use flyback diodes across motors (usually built into L298N)

- Ensure buck converter rated for ≥3A continuous

- Mount L298N on heatsink if running >30 minutes

- Keep gas sensors away from direct fan airflow

- Protect electronics from dust with enclosure

## 🐛 TROUBLESHOOTING GUIDE

| Symptom | Check | Solution |
| --- | --- | --- |
| ADS1115 not detected | I2C wiring | Verify SDA/SCL, try i2cdetect -y 1 |
| ADS1115 at wrong address | ADDR pin | Must connect to GND for 0x48 |
| DHT22 timeout | Power/data pin | Check 5V, verify GPIO 4, try 10kΩ pullup |
| MQ sensors always 0V | Warm-up time | Need 30+ min warmup, 24-48hr ideal |
| PM2.5 always reads 0 | LED control | Verify GPIO 17 → 150Ω → Pin 1 |
| PM2.5 noisy readings | Capacitor | Check 220µF on Pin 5, verify polarity |
| Motors don't run | L298N power | Check 12V connection, enable pins |
| Motors run slow | PWM duty cycle | Increase MOTOR_SPEED in code |
| Fan always on/off | Relay logic | Check GPIO 22, verify relay type |
| Ultrasonic no reading | Echo timeout | Check 5V power, TRIG/ECHO wiring |
| IR sensors inverted | Active level | Adjust potentiometer on IR module |
| Robot moves erratically | Ground loops | Ensure common ground for all components |

## 📊 EXPECTED VOLTAGE READINGS (For Testing)

Normal Operation (Clean Air):

├─ MQ-7 (A0):    0.3-0.6V  (CO sensor)

├─ MQ-135 (A1):  0.2-0.5V  (VOC sensor)

├─ PM2.5 (A2):   0.5-1.0V  (Dust sensor)

└─ DHT22:        Temperature: 20-30°C, Humidity: 40-70%

Poor Air Quality:

├─ MQ-7:         0.8-2.0V  (high CO)

├─ MQ-135:       0.8-2.5V  (high VOC)

└─ PM2.5:        1.5-3.0V  (high dust)

If readings are far outside these ranges, recheck wiring!

## 🎯 FINAL PRE-RUN CHECKLIST

Before running the main code:

- ✅ All power connections verified with multimeter

- ✅ ADS1115 detected at address 0x48

- ✅ All sensors returning reasonable values

- ✅ Motors respond to GPIO control

- ✅ Relay/fan activates with GPIO 22

- ✅ No loose connections or exposed wires

- ✅ Battery fully charged (≥12V)

- ✅ Code uploaded and libraries installed

- ✅ Emergency stop button accessible

- ✅ Testing area clear of obstacles

## 📝 QUICK REFERENCE - GPIO PIN SUMMARY

GPIO  Pin#  Function              Device

----  ----  --------------------  ------------------

2    3     I2C SDA              ADS1115

3    5     I2C SCL              ADS1115

4    7     DHT22 Data           DHT22

7    26    IR Left              IR Sensor

8    24    Ultrasonic Side Echo Ultrasonic

12    32    IR Right             IR Sensor

13    33    Motor Left Enable    L298N ENA

17    11    PM2.5 LED Control    GP2Y1010F48

18    12    Motor Right Enable   L298N ENB

19    35    Motor Left IN1       L298N

20    38    Motor Right IN3      L298N

21    40    Motor Right IN4      L298N

22    15    Relay Fan Control    Relay IN1

23    16    Ultrasonic Front Trig Ultrasonic

24    18    Ultrasonic Front Echo Ultrasonic

25    22    Ultrasonic Side Trig  Ultrasonic

26    37    Motor Left IN2       L298N

Python Code (presented at Sem VII Project B Viva):

"""

Unified Air Quality Monitoring Robot with Automatic Purification

- Robot patrols when AQI ≤ 100 (good air)

- Robot stops and fan activates when AQI > 100 (poor air)

- All sensor readings compensated for temperature and humidity

"""

import time

import board

import busio

import adafruit_ads1x15.ads1115 as ADS

from adafruit_ads1x15.analog_in import AnalogIn

import adafruit_dht

import RPi.GPIO as GPIO

from datetime import datetime

import math

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

# ============ PIN CONFIGURATION ============

# Environmental Sensors

DHT_PIN = 4

PM25_LED_PIN = 17

RELAY_FAN_PIN = 22

# Ultrasonic Sensors

ULTRASONIC_FRONT_TRIG = 23

ULTRASONIC_FRONT_ECHO = 24

ULTRASONIC_SIDE_TRIG = 25

ULTRASONIC_SIDE_ECHO = 8

# IR Sensors

IR_LEFT_PIN = 7

IR_RIGHT_PIN = 12

# L298N Motor Driver

MOTOR_LEFT_EN = 13

MOTOR_LEFT_IN1 = 19

MOTOR_LEFT_IN2 = 26

MOTOR_RIGHT_EN = 18

MOTOR_RIGHT_IN1 = 20

MOTOR_RIGHT_IN2 = 21

# ============ GPIO SETUP ============

# Environmental sensors

GPIO.setup(PM25_LED_PIN, GPIO.OUT)

GPIO.setup(RELAY_FAN_PIN, GPIO.OUT)

GPIO.output(RELAY_FAN_PIN, GPIO.HIGH)  # Fan OFF initially

# Ultrasonic sensors

GPIO.setup(ULTRASONIC_FRONT_TRIG, GPIO.OUT)

GPIO.setup(ULTRASONIC_FRONT_ECHO, GPIO.IN)

GPIO.setup(ULTRASONIC_SIDE_TRIG, GPIO.OUT)

GPIO.setup(ULTRASONIC_SIDE_ECHO, GPIO.IN)

# IR sensors

GPIO.setup(IR_LEFT_PIN, GPIO.IN)

GPIO.setup(IR_RIGHT_PIN, GPIO.IN)

# Motor driver

GPIO.setup(MOTOR_LEFT_EN, GPIO.OUT)

GPIO.setup(MOTOR_LEFT_IN1, GPIO.OUT)

GPIO.setup(MOTOR_LEFT_IN2, GPIO.OUT)

GPIO.setup(MOTOR_RIGHT_EN, GPIO.OUT)

GPIO.setup(MOTOR_RIGHT_IN1, GPIO.OUT)

GPIO.setup(MOTOR_RIGHT_IN2, GPIO.OUT)

# PWM for motor speed control

pwm_left = GPIO.PWM(MOTOR_LEFT_EN, 1000)

pwm_right = GPIO.PWM(MOTOR_RIGHT_EN, 1000)

pwm_left.start(0)

pwm_right.start(0)

# ============ I2C AND SENSOR SETUP ============

i2c = busio.I2C(board.SCL, board.SDA)

ads = ADS.ADS1115(i2c)

mq7_channel = AnalogIn(ads, 0)

mq135_channel = AnalogIn(ads, 1)

pm25_channel = AnalogIn(ads, 2)

dht_sensor = adafruit_dht.DHT22(DHT_PIN, use_pulseio=False)

# ============ CONFIGURATION PARAMETERS ============

# AQI threshold for actions

AQI_THRESHOLD = 100  # Fan ON and Robot STOPS when AQI > 100

# Navigation parameters

OBSTACLE_DISTANCE = 20  # cm

SIDE_DISTANCE = 15      # cm

MOTOR_SPEED = 70        # 0-100 percentage

# Sensor baseline values (calibrate these in clean air)

BASELINE = {

'mq7_clean': 0.4,    # Voltage at clean air for MQ-7

'mq135_clean': 0.3,  # Voltage at clean air for MQ-135

}

# ============ SENSOR READING FUNCTIONS ============

def read_dht22():

"""Read temperature and humidity from DHT22"""

try:

temperature = dht_sensor.temperature

humidity = dht_sensor.humidity

return temperature, humidity

except:

return None, None

def read_mq7():

"""Read MQ-7 CO sensor voltage"""

try:

return mq7_channel.voltage

except:

return None

def read_mq135():

"""Read MQ-135 air quality sensor voltage"""

try:

return mq135_channel.voltage

except:

return None

def read_pm25():

"""Read PM2.5 dust sensor"""

try:

GPIO.output(PM25_LED_PIN, GPIO.LOW)

time.sleep(0.00028)

voltage = pm25_channel.voltage

GPIO.output(PM25_LED_PIN, GPIO.HIGH)

time.sleep(0.00004)

if voltage > 0.6:

dust_density = (voltage - 0.6) * 300

else:

dust_density = 0

return voltage, dust_density

except:

return None, None

# ============ TEMPERATURE/HUMIDITY COMPENSATION ============

def compensate_sensor_reading(raw_value, temp, humidity):

"""

Compensate gas sensor readings for temperature and humidity effects

Reference conditions: 20°C, 65% RH

"""

if temp is None or humidity is None or raw_value is None:

return raw_value

# Reference conditions

ref_temp = 20

ref_humidity = 65

# Temperature compensation (gas sensors show higher response at higher temps)

# 1% change per degree Celsius

temp_factor = 1.0 + ((temp - ref_temp) * 0.01)

# Humidity compensation (moisture interferes with gas detection)

# 0.3% change per %RH

humidity_factor = 1.0 + ((humidity - ref_humidity) * 0.003)

# Apply compensation to get true reading

compensated = raw_value / (temp_factor * humidity_factor)

return compensated

# ============ AQI CALCULATION ============

def calculate_aqi(mq7_voltage, mq135_voltage, pm25_density, temp, humidity):

"""

Calculate combined AQI based on all pollutants with temp/humidity compensation

Returns the MAXIMUM (worst) AQI among all pollutants (EPA method)

"""

if mq7_voltage is None and mq135_voltage is None and pm25_density is None:

return None, None, None, None

# ===== CO SENSOR (MQ-7) PROCESSING =====

co_ppm = None

co_aqi = None

if mq7_voltage is not None:

# Apply temperature and humidity compensation

co_compensated = compensate_sensor_reading(mq7_voltage, temp, humidity)

# Calculate CO concentration in ppm

ratio = co_compensated / BASELINE['mq7_clean']

if ratio >= 1:

co_ppm = 10 ** ((math.log10(ratio) - 0.6) / 0.3)

else:

co_ppm = 0

# Calculate CO AQI (EPA standards)

if co_ppm < 4.4:

co_aqi = (co_ppm / 4.4) * 50

elif co_ppm < 9.4:

co_aqi = 50 + ((co_ppm - 4.4) / 5.0) * 50

elif co_ppm < 12.4:

co_aqi = 100 + ((co_ppm - 9.4) / 3.0) * 50

elif co_ppm < 15.4:

co_aqi = 150 + ((co_ppm - 12.4) / 3.0) * 50

elif co_ppm < 30.4:

co_aqi = 200 + ((co_ppm - 15.4) / 15.0) * 100

else:

co_aqi = 300 + min((co_ppm - 30.4) / 20.0 * 200, 200)

co_aqi = round(co_aqi)

# ===== VOC SENSOR (MQ-135) PROCESSING =====

voc_ratio = None

voc_aqi = None

if mq135_voltage is not None:

# Apply temperature and humidity compensation

voc_compensated = compensate_sensor_reading(mq135_voltage, temp, humidity)

# Calculate VOC ratio

voc_ratio = voc_compensated / BASELINE['mq135_clean']

# Calculate VOC AQI (empirical mapping)

if voc_ratio < 1.2:

voc_aqi = (voc_ratio - 1.0) * 250

elif voc_ratio < 1.5:

voc_aqi = 50 + (voc_ratio - 1.2) * 166.7

elif voc_ratio < 2.0:

voc_aqi = 100 + (voc_ratio - 1.5) * 100

elif voc_ratio < 3.0:

voc_aqi = 150 + (voc_ratio - 2.0) * 50

elif voc_ratio < 5.0:

voc_aqi = 200 + (voc_ratio - 3.0) * 50

else:

voc_aqi = 300 + min((voc_ratio - 5.0) * 40, 200)

voc_aqi = round(voc_aqi)

# ===== PM2.5 SENSOR PROCESSING =====

pm25_aqi = None

if pm25_density is not None:

# PM2.5 is not significantly affected by temp/humidity in measurement

# Calculate PM2.5 AQI (EPA standards)

if pm25_density < 12.0:

pm25_aqi = (pm25_density / 12.0) * 50

elif pm25_density < 35.4:

pm25_aqi = 50 + ((pm25_density - 12.0) / 23.4) * 50

elif pm25_density < 55.4:

pm25_aqi = 100 + ((pm25_density - 35.4) / 20.0) * 50

elif pm25_density < 150.4:

pm25_aqi = 150 + ((pm25_density - 55.4) / 95.0) * 50

elif pm25_density < 250.4:

pm25_aqi = 200 + ((pm25_density - 150.4) / 100.0) * 100

else:

pm25_aqi = 300 + min((pm25_density - 250.4) / 250.0 * 200, 200)

pm25_aqi = round(pm25_aqi)

# ===== COMBINED AQI (EPA METHOD) =====

# Take the MAXIMUM (worst) pollutant AQI

aqi_values = []

if co_aqi is not None:

aqi_values.append(co_aqi)

if voc_aqi is not None:

aqi_values.append(voc_aqi)

if pm25_aqi is not None:

aqi_values.append(pm25_aqi)

if not aqi_values:

return None, None, None, None

combined_aqi = max(aqi_values)

# Cap at 500

if combined_aqi > 500:

combined_aqi = 500

return combined_aqi, co_ppm, voc_ratio, pm25_density

def get_aqi_category(aqi):

"""Get AQI category and color"""

if aqi is None:

return "UNKNOWN", "N/A"

if aqi <= 50:

return "GOOD", "Green"

elif aqi <= 100:

return "MODERATE", "Yellow"

elif aqi <= 150:

return "UNHEALTHY FOR SENSITIVE", "Orange"

elif aqi <= 200:

return "UNHEALTHY", "Red"

elif aqi <= 300:

return "VERY UNHEALTHY", "Purple"

else:

return "HAZARDOUS", "Maroon"

def get_health_message(aqi):

"""Get health advisory based on AQI"""

if aqi is None:

return "Unable to determine air quality"

if aqi <= 50:

return "Air quality is satisfactory"

elif aqi <= 100:

return "Air quality is acceptable"

elif aqi <= 150:

return "Sensitive groups should reduce exposure"

elif aqi <= 200:

return "Everyone should reduce prolonged exertion"

elif aqi <= 300:

return "Health alert: Everyone may experience effects"

else:

return "Health warning: Avoid outdoor exposure"

# ============ FAN CONTROL ============

def control_fan(aqi):

"""

Control purification fan based on AQI only

Fan ON when AQI > 100

Fan OFF when AQI ≤ 100

"""

if aqi is None:

GPIO.output(RELAY_FAN_PIN, GPIO.HIGH)  # Fan OFF if no reading

return "OFF (No AQI)"

if aqi > AQI_THRESHOLD:

GPIO.output(RELAY_FAN_PIN, GPIO.LOW)  # Fan ON

return f"ON (AQI: {aqi})"

else:

GPIO.output(RELAY_FAN_PIN, GPIO.HIGH)  # Fan OFF

return f"OFF (AQI: {aqi})"

# ============ NAVIGATION FUNCTIONS ============

def read_ultrasonic(trig_pin, echo_pin):

"""Read distance from ultrasonic sensor"""

try:

GPIO.output(trig_pin, False)

time.sleep(0.00001)

GPIO.output(trig_pin, True)

time.sleep(0.00001)

GPIO.output(trig_pin, False)

timeout = time.time() + 0.1

while GPIO.input(echo_pin) == 0:

pulse_start = time.time()

if time.time() > timeout:

return None

timeout = time.time() + 0.1

while GPIO.input(echo_pin) == 1:

pulse_end = time.time()

if time.time() > timeout:

return None

pulse_duration = pulse_end - pulse_start

distance = pulse_duration * 17150

distance = round(distance, 2)

if distance < 400:

return distance

else:

return None

except:

return None

def read_ir_sensors():

"""Read IR sensors - returns True if obstacle detected"""

left_obstacle = GPIO.input(IR_LEFT_PIN) == GPIO.LOW

right_obstacle = GPIO.input(IR_RIGHT_PIN) == GPIO.LOW

return left_obstacle, right_obstacle

def stop_motors():

"""Stop both motors"""

pwm_left.ChangeDutyCycle(0)

pwm_right.ChangeDutyCycle(0)

GPIO.output(MOTOR_LEFT_IN1, GPIO.LOW)

GPIO.output(MOTOR_LEFT_IN2, GPIO.LOW)

GPIO.output(MOTOR_RIGHT_IN1, GPIO.LOW)

GPIO.output(MOTOR_RIGHT_IN2, GPIO.LOW)

def move_forward(speed=MOTOR_SPEED):

"""Move both motors forward"""

GPIO.output(MOTOR_LEFT_IN1, GPIO.LOW)

GPIO.output(MOTOR_LEFT_IN2, GPIO.HIGH)

GPIO.output(MOTOR_RIGHT_IN1, GPIO.LOW)

GPIO.output(MOTOR_RIGHT_IN2, GPIO.HIGH)

pwm_left.ChangeDutyCycle(speed)

pwm_right.ChangeDutyCycle(speed)

def move_backward(speed=MOTOR_SPEED):

"""Move both motors backward"""

GPIO.output(MOTOR_LEFT_IN1, GPIO.HIGH)

GPIO.output(MOTOR_LEFT_IN2, GPIO.LOW)

GPIO.output(MOTOR_RIGHT_IN1, GPIO.HIGH)

GPIO.output(MOTOR_RIGHT_IN2, GPIO.LOW)

pwm_left.ChangeDutyCycle(speed)

pwm_right.ChangeDutyCycle(speed)

def turn_left(speed=MOTOR_SPEED):

"""Turn left"""

GPIO.output(MOTOR_LEFT_IN1, GPIO.LOW)

GPIO.output(MOTOR_LEFT_IN2, GPIO.HIGH)

GPIO.output(MOTOR_RIGHT_IN1, GPIO.HIGH)

GPIO.output(MOTOR_RIGHT_IN2, GPIO.LOW)

pwm_left.ChangeDutyCycle(speed)

pwm_right.ChangeDutyCycle(speed)

def turn_right(speed=MOTOR_SPEED):

"""Turn right"""

GPIO.output(MOTOR_LEFT_IN1, GPIO.HIGH)

GPIO.output(MOTOR_LEFT_IN2, GPIO.LOW)

GPIO.output(MOTOR_RIGHT_IN1, GPIO.LOW)

GPIO.output(MOTOR_RIGHT_IN2, GPIO.HIGH)

pwm_left.ChangeDutyCycle(speed)

pwm_right.ChangeDutyCycle(speed)

def navigate(aqi):

"""

Navigation logic based on AQI

Robot MOVES when AQI ≤ 100 (good air)

Robot STOPS when AQI > 100 (poor air, fan purifying)

"""

front_distance = read_ultrasonic(ULTRASONIC_FRONT_TRIG, ULTRASONIC_FRONT_ECHO)

side_distance = read_ultrasonic(ULTRASONIC_SIDE_TRIG, ULTRASONIC_SIDE_ECHO)

ir_left, ir_right = read_ir_sensors()

# Check if robot should move based on AQI

if aqi is None:

stop_motors()

return "STOPPED - No AQI Data", front_distance, side_distance, ir_left, ir_right

if aqi > AQI_THRESHOLD:

stop_motors()

return f"STOPPED - Air Quality Poor (AQI: {aqi})", front_distance, side_distance, ir_left, ir_right

# AQI is acceptable, robot can patrol

action = "PATROLLING"

# Obstacle avoidance logic

if front_distance is not None and front_distance < OBSTACLE_DISTANCE:

stop_motors()

time.sleep(0.2)

move_backward(MOTOR_SPEED)

time.sleep(0.5)

stop_motors()

time.sleep(0.2)

turn_right(MOTOR_SPEED)

time.sleep(0.8)

stop_motors()

action = "AVOIDING - Front Obstacle"

elif side_distance is not None and side_distance < SIDE_DISTANCE:

turn_left(MOTOR_SPEED)

time.sleep(0.3)

stop_motors()

action = "AVOIDING - Side Obstacle"

elif ir_left and ir_right:

stop_motors()

move_backward(MOTOR_SPEED)

time.sleep(0.5)

stop_motors()

action = "AVOIDING - Edge Detected"

elif ir_left:

turn_right(MOTOR_SPEED)

time.sleep(0.3)

stop_motors()

action = "AVOIDING - Left Edge"

elif ir_right:

turn_left(MOTOR_SPEED)

time.sleep(0.3)

stop_motors()

action = "AVOIDING - Right Edge"

else:

move_forward(MOTOR_SPEED)

action = f"PATROLLING - Air Good (AQI: {aqi})"

return action, front_distance, side_distance, ir_left, ir_right

# ============ MAIN FUNCTION ============

def main():

print("="*70)

print("UNIFIED AIR QUALITY ROBOT SYSTEM")

print("="*70)

print("Features:")

print("  - Continuous air quality monitoring with AQI calculation")

print("  - Temperature/Humidity compensation for accurate readings")

print("  - Automatic fan activation when AQI > 100")

print("  - Robot patrol when AQI ≤ 100")

print("  - Robot stops when AQI > 100 for safety")

print("="*70)

print("\nWarming up sensors (30 seconds)...")

time.sleep(30)

print("System ready!\n")

try:

while True:

# Read environmental conditions

temp, humidity = read_dht22()

# Read air quality sensors

mq7_voltage = read_mq7()

mq135_voltage = read_mq135()

pm25_voltage, pm25_density = read_pm25()

# Calculate AQI with temperature/humidity compensation

aqi, co_ppm, voc_ratio, pm25_ug = calculate_aqi(

mq7_voltage, mq135_voltage, pm25_density, temp, humidity

)

# Get AQI information

category, color = get_aqi_category(aqi)

health_msg = get_health_message(aqi)

# Control fan based on AQI only

fan_status = control_fan(aqi)

# Navigate based on AQI

nav_action, front_dist, side_dist, ir_left, ir_right = navigate(aqi)

# Display system status

print("\n" + "="*70)

print(f"Time: {datetime.now().strftime('%Y-%m-%d %H:%M:%S')}")

print("="*70)

print("\nENVIRONMENTAL CONDITIONS:")

if temp is not None and humidity is not None:

print(f"  Temperature: {temp:.1f}°C (Compensating sensors)")

print(f"  Humidity: {humidity:.1f}% (Compensating sensors)")

else:

print("  Temperature/Humidity: Reading failed")

print("\nCOMPENSATED SENSOR READINGS:")

if mq7_voltage is not None:

comp_mq7 = compensate_sensor_reading(mq7_voltage, temp, humidity)

print(f"  MQ-7 (CO): {mq7_voltage:.3f}V → {comp_mq7:.3f}V (compensated)", end="")

if co_ppm is not None:

print(f" → ~{co_ppm:.1f} ppm CO")

else:

print()

else:

print("  MQ-7 (CO): Reading failed")

if mq135_voltage is not None:

comp_mq135 = compensate_sensor_reading(mq135_voltage, temp, humidity)

print(f"  MQ-135 (VOC): {mq135_voltage:.3f}V → {comp_mq135:.3f}V (compensated)", end="")

if voc_ratio is not None:

print(f" → Ratio: {voc_ratio:.2f}")

else:

print()

else:

print("  MQ-135 (VOC): Reading failed")

if pm25_voltage is not None and pm25_density is not None:

print(f"  PM2.5 (Dust): {pm25_voltage:.3f}V → {pm25_density:.1f} μg/m³")

else:

print("  PM2.5 (Dust): Reading failed")

print("\n" + "="*70)

print("AIR QUALITY INDEX (AQI):")

print("="*70)

if aqi is not None:

print(f"  AQI: {aqi} - {category} ({color})")

print(f"  Health: {health_msg}")

print(f"  Threshold: {AQI_THRESHOLD} (Fan/Robot trigger)")

else:

print("  AQI: Unable to calculate")

print("="*70)

print(f"\nPURIFICATION FAN: {fan_status}")

if aqi is not None:

if aqi > AQI_THRESHOLD:

print("  → Actively purifying air")

else:

print("  → Air quality acceptable")

print(f"\nROBOT STATUS: {nav_action}")

print(f"  Front: {front_dist if front_dist else 'N/A'} cm")

print(f"  Side: {side_dist if side_dist else 'N/A'} cm")

print(f"  IR Left: {'OBSTACLE' if ir_left else 'CLEAR'}")

print(f"  IR Right: {'OBSTACLE' if ir_right else 'CLEAR'}")

if aqi is not None:

if aqi <= AQI_THRESHOLD:

print(f"  → Mode: PATROL (AQI ≤ {AQI_THRESHOLD})")

else:

print(f"  → Mode: STATIONARY PURIFICATION (AQI > {AQI_THRESHOLD})")

print("="*70)

time.sleep(5)

except KeyboardInterrupt:

print("\n\nSystem stopped by user")

except Exception as e:

print(f"\n\nError: {e}")

import traceback

traceback.print_exc()

finally:

stop_motors()

pwm_left.stop()

pwm_right.stop()

GPIO.output(RELAY_FAN_PIN, GPIO.HIGH)

GPIO.cleanup()

print("\nGPIO cleaned up. System shutdown complete.")

if __name__ == "__main__":

main()